QC parameters in Garching

It is foreseen to implement in the MIDI pipeline the following recipes:

- Detector Calibrations (cf imaging instruments)

  - Master flat

  - Master Dark

  - Bad pixels

- Wavelength calibration (cf spectroscopic instruments)

  - dispersion relation

  - spectral sensitivity

- Zero Point (cf photometric instruments)

- Aperture Mask determination

This will define the position of the images/spectra on the detector.

A weighted mask will be extracted from the photometric data to be applied to the fringe data

  - non dispersed fringes

  - dispersed fringes

- Estimate uncalibrated visibility

The mask defined will be applied, the raw visibility of the fringes will be measured

- Estimate Transfer function  : recipe TF

If the object observed is an astronomical calibrator, the TF will be calculated from the raw visibility and the estimated visibility

- Estimate Calibrated Visibility : recipe Visibility

If the object observed is a science target and if a TF is available, a calibrated visibility will be calculated

For each product, some of the PRO keywords will be common

HIERARCH ESO PRO TYPE

HIERARCH ESO PRO CATG

HIERARCH ESO PRO ARCFILE

HIERARCH ESO PRO PIPEDATE

HIERARCH ESO PRO VERSION

HIERARCH ESO PRO PIPEFILE

HIERARCH ESO PRO DID

HIERARCH ESO PRO QC ID

HIERARCH ESO PRO QC STATUS

HIERARCH ESO PRO QC CERTIF

HIERARCH ESO PRO QC DATE

For each of the recipes they are additional PRO keywords and a set of QC parameters

to be defined/measured by the pipeline

# Reduce Fringe

The raw visibility is calculated and normalized with the photometry

### Specific PRO keywords

HIERARCH ESO PRO REC1 RAW1 NAME

HIERARCH ESO PRO REC1 RAW1 CATG

HIERARCH ESO PRO REC1 RAW2 NAME

HIERARCH ESO PRO REC1 RAW2 CATG

HIERARCH ESO PRO REC1 RAW3 NAME

HIERARCH ESO PRO REC1 RAW3 CATG

HIERARCH ESO PRO REC1 RAW4 NAME

HIERARCH ESO PRO REC1 RAW4 CATG

….

HIERARCH ESO PRO REC1 PHOT1 NAME

HIERARCH ESO PRO REC1 PHOT1 CATG

HIERARCH ESO PRO REC1 PHOT2 NAME

HIERARCH ESO PRO REC1 PHOT2 CATG

### QC Keywords:

Number of scans processed

HIERARCH ESO QC SCANS PROCESSED

Number of scans rejected

HIERARCH ESO QC SCANS REJECTED

Uvw coordinates

HIERARCH ESO QC BASELINE UVW1

HIERARCH ESO QC BASELINE UVW2

HIERARCH ESO QC BASELINE UVW3

Baseline length

HIERARCH ESO QC BASELINE LENGTH

Baseline projected angle

HIERARCH ESO QC BASELINE ANGLE

Number of photons in both channels (0 and 1) for beams A and B (extracted from photometry)

HIERARCH ESO QC REDUCE PHOTA0

HIERARCH ESO QC REDUCE DPHOTA0

HIERARCH ESO QC REDUCE PHOTA1

HIERARCH ESO QC REDUCE DPHOTA1

HIERARCH ESO QC REDUCE PHOTB0

HIERARCH ESO QC REDUCE DPHOTB0

HIERARCH ESO QC REDUCE PHOTB1

HIERARCH ESO QC REDUCE DPHOTB1

Ratio of the number of photons between 0 and 1 for beams A and B (extracted from photometry)

HIERARCH ESO QC REDUCE PHOTAB0

HIERARCH ESO QC REDUCE PHOTAB1

Rms between the chopping cycles (extracted from photometry)

HIERARCH ESO QC REDUCE RMS CHOPA0

HIERARCH ESO QC REDUCE RMS CHOPA1

HIERARCH ESO QC REDUCE RMS CHOPB0

HIERARCH ESO QC REDUCE RMS CHOPB1

uncorrelated flux

HIERARCH ESO QC REDUCE UNCORR FLUX

fringe noise threshold

HIERARCH ESO QC REDUCE NOISE THRES

fringe signal thershold

HIERARCH ESO QC REDUCE FRINGE THRES

fringe mean signal

HIERARCH ESO QC REDUCE FRINGE AVG

fringe mean noise

HIERARCH ESO QC REDUCE NOISE AVG

fringe rms signal

HIERARCH ESO QC REDUCE FRINGE RMS

fringe rms noise

HIERARCH ESO QC REDUCE NOISE RMS

fringe SNR

HIERARCH ESO QC REDUCE FRINGE SNR

correlated flux

HIERARCH ESO QC REDUCE CORR FLUX

Uncalibrated visibilitie averaged

HIERARCH ESO QC REDUCE MU

Associated error

HIERARCH ESO QC REDUCE EMU

Uncalibrated visibility Channel1

HIERARCH ESO QC REDUCE MU 1

Asssociated error

HIERARCH ESO QC REDUCE EMU1

Uncalibrated visibility Channel2

HIERARCH ESO QC REDUCE MU2

Associated error

HIERARCH ESO QC REDUCE EMU2

OPD rate

HIERARCH ESO QC REDUCE OPD RATE

Fringe rate

HIERARCH ESO QC REDUCE FRINGE RATE

mask file used

HIERARCH ESO QC REDUCE MASKi

# Vis

Applied TF

HIERARCH ESO QC VISIBILITY AT1

HIERARCH ESO QC VISIBILITY DAT1

HIERARCH ESO QC VISIBILITY AT2

HIERARCH ESO QC VISIBILITY DAT2

Calibrated Visibilities

HIERARCH ESO QC VISIBILITY V0

HIERARCH ESO QC VISIBILITY DV0

HIERARCH ESO QC VISIBILITY V1

HIERARCH ESO QC VISIBILITY DV1

HIERARCH ESO QC VISIBILITY V22

HIERARCH ESO QC VISIBILITY DV2

# TF

TF channel1

HIERARCH ESO QC ESTIMATETF T1

Error TF channel 1

HIERARCH ESO QC ESTIMATETF DT1

CO TF channel1

HIERARCH ESO QC ESTIMATETF COT1

error coTF channel1

HIERARCH ESO QC ESTIMATETF DCOT1

TF channel2

HIERARCH ESO QC ESTIMATETF T2

Error TF channel 2

HIERARCH ESO QC ESTIMATETF DT2

CO TF channel2

HIERARCH ESO QC ESTIMATETF COT2

error coTF channel2

HIERARCH ESO QC ESTIMATETF DCOT2

Calibrator position in the list

HIERARCH ESO QC ESTIMATETF CALIB POS

Calibrator name

HIERARCH ESO QC ESTIMATETF CALIB NAME

Calibrator RA

HIERARCH ESO QC ESTIMATETF CALIB RA

Calibrator DEC

HIERARCH ESO QC ESTIMATETF CALIB DEC

Dist between keywords CALIB RA – CALIB DEC and primary headers RA and DEC

HIERARCH ESO QC ESTIMATETF CALIB DIST

Calibrator estimated diameter

HIERARCH ESO QC ESTIMATETF CALIB DIAM

Error estimated diam

HIERARCH ESO QC ESTIMATETF CALIB EDIAM

Flag form the vlti list

HIERARCH ESO QC ESTIMATETF CALIB FLAG

Expected Visibility

HIERARCH ESO QC ESTIMATETF CALIB VIS

Error expected Visibility

HIERARCH ESO QC ESTIMATETF CALIB EVIS

# Determination of the Masks

## In dispersed mode

Name of the mask produced

HIERARCH ESO ESO PRO MASK

SNR

HIERARCH ESO QC SNRPA1

HIERARCH ESO QC SNRPA2

HIERARCH ESO QC SNRPB1

HIERARCH ESO QC SNRPB2

ncounts

HIERARCH ESO QC SPECPHOT NCOUNTA1

ncounts_min

HIERARCH ESO QC SPECPHOT NCOUNTMINA1

ncount_max

HIERARCH ESO QC SPECPHOT NCOUNTMAXA1

ncount_median

HIERARCH ESO QC SPECPHOT NCOUNTMEDA1

ncount_rms

HIERARCH ESO QC SPECPHOT NCOUNTRMSA1

image quality

HIERARCH ESO QC SPECPHOT IMGQUA

beam overlap

HIERARCH ESO QC SPECPHOT OVERLAP

mask overlap

HIERARCH ESO QC SPECPHOT MASK OVERLAPi

beam jitter

HIERARCH ESO QC SPECPHOT BEAM JITTERi

rms beam jitter

HIERARCH ESO QC SPECPHOT BEAM JITTERRMSi

median

HIERARCH ESO QC SPECPHOT BEAM JITTERMED

lambda_min

HIERARCH ESO QC SPECPHOT WLENMIN

lambda_max

HIERARCH ESO QC SPECPHOT WLENMAX

wavelength of the sdt source

HIERARCH ESO QC SPECPHOT SDT WLEN

transmission channel I for the SDT WLEN

HIERARCH ESO QC SPECPHOT EFFi

source name

HIERARCH ESO QC SPECPHOT  SDT

source sptype

HIERARCH ESO QC SPECPHOT  SDT SP TYPE

source effective temperature

HIERARCH ESO QC SPECPHOT  SDT TEFF

FWHM average at sdt wlen

HIERARCH ESO QC SPECPHOT FWHMAVG

FWHM rms at sdt wlen

HIERARCH ESO QC SPECPHOT FWHMRMS

## In Non-dispersed Mode

Name of the mask produced

HIERARCH ESO ESO PRO MASK

SNR

HIERARCH ESO QC SNRPA1

HIERARCH ESO QC SNRPA2

HIERARCH ESO QC SNRPB1

HIERARCH ESO QC SNRPB2

ncounts

HIERARCH ESO QC SPECPHOT NCOUNTA1

ncounts_min

HIERARCH ESO QC SPECPHOT NCOUNTMINA1

ncount_max

HIERARCH ESO QC SPECPHOT NCOUNTMAXA1

ncount_median

HIERARCH ESO QC SPECPHOT NCOUNTMEDA1

ncount_rms

HIERARCH ESO QC SPECPHOT NCOUNTRMSA1

image quality

HIERARCH ESO QC SPECPHOT IMGQUA

beam overlap

HIERARCH ESO QC SPECPHOT OVERLAP

mask overlap

HIERARCH ESO QC SPECPHOT MASK OVERLAPi

beam jitter

HIERARCH ESO QC SPECPHOT BEAM JITTERi

rms beam jitter

HIERARCH ESO QC SPECPHOT BEAM JITTERRMSi

median

HIERARCH ESO QC SPECPHOT BEAM JITTERMED

# Sky background

ncounts

HIERARCH ESO QC SKYBACK NCOUNT

ncounts_min

HIERARCH ESO QC SKYBACK NCOUNTMIN

ncount_max

HIERARCH ESO QC SKYBACK NCOUNTMAX

ncount_median

HIERARCH ESO QC SKYBACK SKYMED

ncount_rms

HIERARCH ESO QC SKYBACK SKYRMS

ncount_av

HIERARCH ESO QC SKYBACK SKYAV

# Spectral sensitivity

baselinelenght

HIERARCH ESO QC BASELINE LENGTH

baselineangle

HIERARCH ESO QC BASELINE ANGLE

lambda_min

HIERARCH ESO QC SPEC SENS WLENMIN

lambda_max

HIERARCH ESO QC SPEC SENS WLENMAX

central wavelength

HIERARCH ESO QC SPEC SENS WLEN

transmission channel i

HIERARCH ESO QC SPEC SENS EFFi

source name

HIERARCH ESO QC SPEC SENS SDT

source sptype

HIERARCH ESO QC SPEC SENS SDT SP TYPE

source effective temperature

HIERARCH ESO QC SPEC SENS SDT TEFF

# Bad pixels map

number of hot pixels

HIERARCH ESO QC FLAT NHOTPIX

fixed pattern noise

HIERARCH ESO QC FLAT FPN

sensitivity

HIERARCH ESO QC FLAT SENS

number counts

HIERARCH ESO QC FLAT NCOUNTS

photon noise

HIERARCH ESO QC FLAT PHN

# Detector RON

read_out_noise

HIERARCH ESO QC DARK RON

read_out_noise rms

HIERARCH ESO QC DARK RON RMS

read_out_noise average

HIERARCH ESO QC DARK RON AVG

# dispersion relation

lambda_min

HIERARCH ESO QC WAVE WLENMIN

lambda_max

HIERARCH ESO QC WAVE WLENMAX

central wavelength

HIERARCH ESO QC WAVE WLEN

number lines used

HIERARCH ESO QC WAVE NLIN

FWHM average

HIERARCH ESO QC WAVE FWHMAVG

FWHM rms

HIERARCH ESO QC WAVE FWHMRMS

source name

HIERARCH ESO QC WAVE DISP SRC

disp relation

HIERARCH ESO QC WAVE DISPC01

disp relation

HIERARCH ESO QC WAVE DISPC02

disp relation

HIERARCH ESO QC WAVE DISPC03